# Feature Extraction Process (From CSI to Range–Doppler Features)

This section describes how Channel State Information (CSI) from Wi-Fi packets is processed into numerical Range–Doppler (RD) features for deep learning–based environment classification.

## 1. Sliding-Window Segmentation

Each CSI .h5 file contains complex CSI samples for multiple Wi-Fi frames.
A sliding window of win = 256 frames is used, with an overlap of 50% (inc = 128 frames).

For each window:

- CSI data csi∈ℂ^{T×108} and corresponding timestamps are extracted.

- Only the first 52 LLTF subcarriers (excluding DC) are used:

csi52∈CT×52

## 2. RSSI-Based Amplitude Calibration

Each CSI frame is scaled according to its measured RSSI (rssi_dbm) to maintain a consistent amplitude level across time:

scale=10(rssi_dbm/10)mean(∣csi∣2)

Then:

csiscaled=csi×scale×103

This step normalizes power variations due to AGC and transmission power differences.

## 3. DC Subcarrier Reconstruction

The Wi-Fi LLTF contains 52 non-zero subcarriers (−26…−1, +1…+26).
The DC component (0) is missing and filled as:

CSI0=0.5×(CSI-1+CSI+1)

This yields a 53-subcarrier CSI matrix:

csi53∈CT×53

## 4. Power Computation

The instantaneous CSI power is computed as:

CSI_PWR=∣csi53∣2=R(csi53⋅csi53*)

Resulting in a real-valued power matrix of shape (T, 53).

## 5. Time-Axis Uniformization

Because ESP32 timestamps are slightly irregular, the time axis is re-sampled to a uniform grid before Doppler analysis:

- Compute relative time in seconds:

trel=t-t0

- Estimate sampling frequency:

fs=N-1trel[-1]-trel[0]

- Create a uniform time vector tuand interpolate each subcarrier column using linear interpolation:

CSI_PWR_u(tu)=Interp(trel,CSI_PWR)

- Subtract the mean per column to reduce DC leakage.

This produces a uniformly sampled power matrix CSI_PWR_u∈RTu×53.

## 6. Range Transformation (Frequency Domain FFT)

To convert frequency-domain variations into spatial (range) information, a zoom-FFT is applied across subcarriers (axis = 1):

Range_FFT=ZoomFFT(CSI_PWR_u,fmin=0,fmax=32 m,M=128)

This maps subcarrier phase slope to propagation distance (0–32 m).

## 7. Doppler Transformation (Time Domain FFT)

A second zoom-FFT is applied along the time axis (axis = 0) using the estimated sampling rate:

Doppler_FFT=ZoomFFT(Range_FFT,fmin=-150 Hz,fmax=150 Hz,M=256)

This converts temporal fluctuations into motion-induced Doppler frequencies.

## 8. Magnitude Feature Construction

The complex Doppler–Range response is converted into magnitude:

Feature=∣Doppler_FFT∣

The resulting 2-D array represents power intensity across range bins (horizontal) and Doppler bins (vertical):

Feature∈R256×128
Each matrix is saved as a .npy file for downstream deep-learning training.